WiFi Training Program

Assignment Questions - Module 1

1. In which OSL layer the Wi-Fi standard/protocol fits.

Since Wi-fi is a secure, wireless medium of data transmission, it operates in the Physical and Data-Link Layer of the OSI model.

- In Wi-fi, the data is transmitted over the air using radio waves and several modulation techniques. So, the data transmission part of Wi-fi falls under the physical layer of the OSI model.

- Wi-fi does Frame Formatting, Addressing, Error Detection and Correction (CRC) and uses CSMA/CA for collision avoidance, typically highlighting the MAC sublayer functions of the Data-Link Layer of the OSI model.

- Finally, Wi-fi Does flow control and uses protocols like IP, ARP etc for proper delivery of data which is the LLC sublayer of the data link layer.

2. Can you share the Wi-Fi devices that you are using day to day life, share that device's wireless

capability/properties after connecting to network. Match your device to corresponding Wi-Fi

Generations based on properties.

Some of the Devices I use day-to-day in my life include:

1. Smartphone:

- Standard: Wi-Fi 6 (IEEE802.11ax)

- Frequency: 5 GHz

- Channel width: 20 / 40 / 80 MHz

- Link speed: 25 Mbps – 30 Mbps

- MIMO: 2×2 MIMO

- Security: WPA2 / WPA3

2. Laptop:

- Standard: Wi-Fi 5 (IEEE802.11ac)

- Frequency: 5 GHz (preferred) + 2.4 GHz fallback

- Channel width: up to 80 MHz / 160 MHz

- Link speed: 25 Mbps – 30 Mbps

- MIMO: 2×2 or 3×3

- Modulation: 256-QAM (Wi-Fi 5), 1024-QAM (Wi-Fi 6)

3. Smart TV:

- Standard: Wi-Fi 4 (IEEE802.11n)

- Frequency: Mostly 2.4 GHz

- Channel width: 20 / 40 MHz

- Link speed: 30 Mbps

- MIMO: usually 1×1 or 2×2

3. what is BSS and ESS?

A Basic Service Set consists of one Access point and Multiple client devices. This Wi-fi network is identified by SSID and BSSID. Whereas, Extended Service Set consists of Multiple Aps connected by a wired lan. Every AP is connected to multiple client devices. All the Aps share a common SSID. ESS enables easy roaming without getting disconnected as while moving, the device automatically gets connected to another AP.

4. what are the basic functionalities of Wi-Fi Access point?

Functionalities of AP:

- Converts wired Ethernet → wireless signals

- Creates the Wi-Fi network (SSID)

- Handles Authentication using Security Protocols like WPA2/WPA3.

- Does frame forwarding and bridging by connecting wireless clients with wired switched/ routers.

- Does Media Access control for avoiding collisions using CSMA/CA protocol.

- Offers roaming support by helping devices to switch between Aps in ESS.

- Controls channel parameters like Bandwidth and Data transmission Rate.

5. What are Differences between Bridge mode and Repeater mode.

| Bridge Mode | Repeater Mode |
| --- | --- |
| Connect two Networks | Extend Network Coverage of a given wi-fi network. |
| No clients are connected in this mode of operation. | Clients are present. |
| No major Loss is incured | Loss might occur and re-transmission may be required |
| Point-to-Point Link | Signal Re-Transmission |
| Usually used in a building-to-building link. | Used for eliminating dead zones where signal gets completely lost. |

6. what are the differences between 802.11a and 802.11b.

7. Configure your modem/hotspot to operate only in 2.4Ghz and connect your laptop/Wi-Fi device, and capture the capability/properties in your Wi-Fi device. Repeat the same in 5Ghz and tabulate all the differences you observed during this

8. What is the difference between IEEE and WFA

9. List down the type of Wi-Fi internet connectivity backhaul, share your home/college's wireless internet connectivity backhaul name and its properties.

10. List down the Wi-Fi topologies and use cases of each one.